%% Step 0: Initialization

channelID = CHANNEL_ID;

readAPIKey = READ_API_KEY';

numPoints = 500;

%% Step 1: Read 3-Phase Data from ThingSpeak

mainData = thingSpeakRead(channelID, 'Fields', [1 2 3], 'ReadKey', readAPIKey, 'NumPoints', numPoints);

loadData = thingSpeakRead(channelID, 'Fields', [4 5 6], 'ReadKey', readAPIKey, 'NumPoints', numPoints);

timestamps = thingSpeakRead(channelID, 'Fields', 1, 'ReadKey', readAPIKey, 'NumPoints', numPoints, 'OutputFormat', 'timetable').Timestamps;

% Handle missing values

mainData = fillmissing(mainData, 'linear');

loadData = fillmissing(loadData, 'linear');

%% Step 2: Feature Engineering

mainSum = sum(mainData, 2);  % Total current at Main

loadSum = sum(loadData, 2);  % Total current at Load

diff = mainSum - loadSum;    % Difference (possible leakage or theft)

relDiff = diff ./ mainSum;   % Relative difference (helps normalize variations)

% Feature matrix

X = [mainData, loadData, diff, relDiff];

Xnorm = normalize(X);        % Normalize for ML

%% Step 3: Isolation Forest (Requires MATLAB R2023a+)

try

modelIF = iforest(Xnorm, 'ContaminationFraction', 0.05);

[labelsIF, scoreIF] = anomalyScore(modelIF, Xnorm);

catch

warning('Isolation Forest not available in this MATLAB version.');

labelsIF = zeros(numPoints, 1);

scoreIF = zeros(numPoints, 1);

end

%% Step 4: One-Class SVM

svmModel = fitcsvm(Xnorm, ones(size(Xnorm,1),1), ...

'KernelScale','auto', ...

'Standardize',true, ...

'OutlierFraction',0.05);

[~, scoreSVM] = predict(svmModel, Xnorm);

labelsSVM = scoreSVM < 0;  % Outliers marked as 1

%% Step 5: K-Means Clustering (2 clusters: normal, abnormal)

[idxKMeans, C] = kmeans(Xnorm, 2, 'Replicates', 5);

% Label the smaller cluster as anomaly

clusterSizes = histcounts(idxKMeans, 2);

[~, anomalyCluster] = min(clusterSizes);

labelsKM = idxKMeans == anomalyCluster;

%% Step 6: Statistical Thresholding (baseline)

thresh = 0.15;  % Relative difference threshold (tunable)

labelsThresh = abs(relDiff) > thresh;

%% Step 7: Combine and Visualize Results

figure;

subplot(4,1,1);

plot(timestamps, diff); hold on;

scatter(timestamps(labelsIF==1), diff(labelsIF==1), 'r*');

title('Isolation Forest Anomalies'); ylabel('Δ Current (A)');

subplot(4,1,2);

plot(timestamps, diff); hold on;

scatter(timestamps(labelsSVM==1), diff(labelsSVM==1), 'ko');

title('One-Class SVM Anomalies'); ylabel('Δ Current (A)');

subplot(4,1,3);

plot(timestamps, diff); hold on;

scatter(timestamps(labelsKM==1), diff(labelsKM==1), 'gx');

title('K-Means Anomalies'); ylabel('Δ Current (A)');

subplot(4,1,4);

plot(timestamps, diff); hold on;

scatter(timestamps(labelsThresh==1), diff(labelsThresh==1), 'ms');

title('Statistical Threshold Anomalies'); ylabel('Δ Current (A)');

xlabel('Timestamp');

sgtitle('Electricity Theft Detection using Machine Learning');

%% Step 8: Print Summary

fprintf('\n--- Anomaly Summary ---\n');

fprintf('Isolation Forest: %d anomalies\n', sum(labelsIF));

fprintf('One-Class SVM:    %d anomalies\n', sum(labelsSVM));

fprintf('K-Means:          %d anomalies\n', sum(labelsKM));

fprintf('Thresholding:     %d anomalies\n', sum(labelsThresh));